Gateway Node.


import network

import espnow

import urequests

import json

import time

import ubinascii

from machine import Pin, time_pulse_us

# ---------------- CONFIG ----------------

WIFI_SSID = "x"

WIFI_PASS = "duckduckduck"

SERVER_HOST = "192.168.16.112"

SERVER_PORT = 8000

DJANGO_URL = "http://{}:{}/api/data/receive/".format(SERVER_HOST, SERVER_PORT)

# Ultrasonic config

ULTRASONIC_NODE_ID = "us01"

ULTRASONIC_INTERVAL_MS = 3000

# Ultrasonic 1 pins

TRIG1_PIN = 5

ECHO1_PIN = 18

# Ultrasonic 2 pins

TRIG2_PIN = 19

ECHO2_PIN = 21

# ----------------------------------------

# ---------- WiFi Setup ----------

sta = network.WLAN(network.STA_IF)

sta.active(True)

sta.config(dhcp_hostname="Gateway")

sta.connect(WIFI_SSID, WIFI_PASS)

print("Connecting to WiFi...")

while not sta.isconnected():

time.sleep(1)

try:

sta.config(pm=0xa11140)

except:

pass

print("IP Address:", sta.ifconfig()[0])

mac_bytes = sta.config("mac")

print("Gateway MAC:", ubinascii.hexlify(mac_bytes, b":").decode())

print("===========================\n")

# ---------- ESP-NOW Setup ----------

e = espnow.ESPNow()

e.active(True)

# ---------- Ultrasonic Setup ----------

trig1 = Pin(TRIG1_PIN, Pin.OUT, pull=None)

echo1 = Pin(ECHO1_PIN, Pin.IN, pull=None)

trig2 = Pin(TRIG2_PIN, Pin.OUT, pull=None)

echo2 = Pin(ECHO2_PIN, Pin.IN, pull=None)

trig1.value(0)

trig2.value(0)

def read_ultrasonic_cm(trig, echo):

trig.value(0)

time.sleep_us(2)

trig.value(1)

time.sleep_us(10)

trig.value(0)

try:

duration = time_pulse_us(echo, 1, 30000)

except OSError:

print('la muji')

return None

if duration <= 0:

print('barbad muji')

return None

return round((duration * 0.0343) / 2, 2)

def forward_to_django(payload):

try:

headers = {"Content-Type": "application/json"}

r = urequests.post(

DJANGO_URL,

data=json.dumps(payload),

headers=headers

)

r.close()

print(">> Forwarded to Server:", payload)

except Exception as err:

print("!! HTTP Error:", err)

# ---------- Main Loop ----------

print("Gateway running...")

last_ultrasonic_time = 0

while True:

now = time.ticks_ms()

# 1. Receive ESP-NOW packets

try:

host, msg = e.irecv(0)

if msg:

try:

data = json.loads(msg.decode())

print("<< Received ESP-NOW:", data)

forward_to_django(data)

except ValueError:

print("Received non-JSON ESP-NOW payload")

except Exception:

pass

# 2. Read both ultrasonic sensors and average

if time.ticks_diff(now, last_ultrasonic_time) > ULTRASONIC_INTERVAL_MS:

d1 = read_ultrasonic_cm(trig1, echo1)

d2 = read_ultrasonic_cm(trig2, echo2)

valid_readings = []

if d1 is not None:

valid_readings.append(d1)

if d2 is not None:

valid_readings.append(d2)

if valid_readings:

avg_distance = round(

sum(valid_readings) / len(valid_readings),

2

)

payload = {

"nodeid": ULTRASONIC_NODE_ID,

"value": avg_distance,

#               "raw": {

#                  "sensor1": d1,

#                 "sensor2": d2

#         }

}

forward_to_django(payload)

else:

print("Ultrasonic read failed on both sensors")

last_ultrasonic_time = now

time.sleep_ms(10)

----------------------------------

---------------

-----------------

-----------------

-----------------

-----------------

Actuator node

import network

import espnow

import urequests

import json

import time

import ubinascii

from machine import Pin, time_pulse_us

# ---------------- CONFIG ----------------

WIFI_SSID = "x"

WIFI_PASS = "duckduckduck"

SERVER_HOST = "192.168.16.112"

SERVER_PORT = 8000

DJANGO_URL = "http://{}:{}/api/data/receive/".format(SERVER_HOST, SERVER_PORT)

# Ultrasonic config

ULTRASONIC_NODE_ID = "us01"

ULTRASONIC_INTERVAL_MS = 3000

# Ultrasonic 1 pins

TRIG1_PIN = 5

ECHO1_PIN = 18

# Ultrasonic 2 pins

TRIG2_PIN = 19

ECHO2_PIN = 21

# ----------------------------------------

# ---------- WiFi Setup ----------

sta = network.WLAN(network.STA_IF)

sta.active(True)

sta.config(dhcp_hostname="Gateway")

sta.connect(WIFI_SSID, WIFI_PASS)

print("Connecting to WiFi...")

while not sta.isconnected():

time.sleep(1)

try:

sta.config(pm=0xa11140)

except:

pass

print("IP Address:", sta.ifconfig()[0])

mac_bytes = sta.config("mac")

print("Gateway MAC:", ubinascii.hexlify(mac_bytes, b":").decode())

print("===========================\n")

# ---------- ESP-NOW Setup ----------

e = espnow.ESPNow()

e.active(True)

# ---------- Ultrasonic Setup ----------

trig1 = Pin(TRIG1_PIN, Pin.OUT, pull=None)

echo1 = Pin(ECHO1_PIN, Pin.IN, pull=None)

trig2 = Pin(TRIG2_PIN, Pin.OUT, pull=None)

echo2 = Pin(ECHO2_PIN, Pin.IN, pull=None)

trig1.value(0)

trig2.value(0)

def read_ultrasonic_cm(trig, echo):

trig.value(0)

time.sleep_us(2)

trig.value(1)

time.sleep_us(10)

trig.value(0)

try:

duration = time_pulse_us(echo, 1, 30000)

except OSError:

print('la muji')

return None

if duration <= 0:

print('barbad muji')

return None

return round((duration * 0.0343) / 2, 2)

def forward_to_django(payload):

try:

headers = {"Content-Type": "application/json"}

r = urequests.post(

DJANGO_URL,

data=json.dumps(payload),

headers=headers

)

r.close()

print(">> Forwarded to Server:", payload)

except Exception as err:

print("!! HTTP Error:", err)

# ---------- Main Loop ----------

print("Gateway running...")

last_ultrasonic_time = 0

while True:

now = time.ticks_ms()

# 1. Receive ESP-NOW packets

try:

host, msg = e.irecv(0)

if msg:

try:

data = json.loads(msg.decode())

print("<< Received ESP-NOW:", data)

forward_to_django(data)

except ValueError:

print("Received non-JSON ESP-NOW payload")

except Exception:

pass

# 2. Read both ultrasonic sensors and average

if time.ticks_diff(now, last_ultrasonic_time) > ULTRASONIC_INTERVAL_MS:

d1 = read_ultrasonic_cm(trig1, echo1)

d2 = read_ultrasonic_cm(trig2, echo2)

valid_readings = []

if d1 is not None:

valid_readings.append(d1)

if d2 is not None:

valid_readings.append(d2)

if valid_readings:

avg_distance = round(

sum(valid_readings) / len(valid_readings),

2

)

payload = {

"nodeid": ULTRASONIC_NODE_ID,

"value": avg_distance,

#               "raw": {

#                  "sensor1": d1,

#                 "sensor2": d2

#         }

}

forward_to_django(payload)

else:

print("Ultrasonic read failed on both sensors")

last_ultrasonic_time = now

time.sleep_ms(10)

KULLO 02

import network

import espnow

import time

import json

from machine import ADC, Pin

# ---------------- CONFIG ----------------

NODE_ID = "002"

DEVICE_NAME = "Kullo2"

SENSOR_PIN = 34

READ_INTERVAL = 5000

# !!! UPDATE THESE TWO LINES FROM GATEWAY OUTPUT !!!

GATEWAY_MAC = b'\xcc\x50\xe3\x93\x0f\x58'  # Example: b'\x24\x6f...'

WIFI_CHANNEL = 3  # MUST match the Gateway's channel

# ----------------------------------------

# Setup WLAN

sta = network.WLAN(network.STA_IF)

sta.active(True)

sta.config(dhcp_hostname=DEVICE_NAME)

# Set the channel to match Gateway

sta.config(channel=WIFI_CHANNEL)

sta.disconnect() # Ensure we are not connected to any AP

print(f"Node started on Channel {WIFI_CHANNEL}")

# Setup ESP-NOW

e = espnow.ESPNow()

e.active(True)

try:

e.add_peer(GATEWAY_MAC)

except OSError:

print("Peer already exists or invalid MAC")

# Setup Sensor

adc = ADC(Pin(SENSOR_PIN))

adc.atten(ADC.ATTN_11DB)

adc.width(ADC.WIDTH_12BIT)

def read_humidity():

raw = adc.read()

# Map 0-4095 to 0-100%

return round((raw / 4095.0) * 100, 2)

def send_data(val):

payload = {

"nodeid": NODE_ID,

"value": val

}

msg = json.dumps(payload)

try:

e.send(GATEWAY_MAC, msg)

print(f"Sent: {msg}")

except OSError as err:

if err.args[0] == 116: # ESP_ERR_ESPNOW_NOT_FOUND

print("Error: Gateway not found (Check Channel/MAC)")

else:

print("Send Error:", err)

while True:

hum = read_humidity()

send_data(hum)

time.sleep_ms(READ_INTERVAL)

KULLO 01

import network

import espnow

import time

import json

from machine import ADC, Pin

# ---------------- CONFIG ----------------

NODE_ID = "001"

DEVICE_NAME = "Kullo1"

SENSOR_PIN = 34

READ_INTERVAL = 5000

# !!! UPDATE THESE TWO LINES FROM GATEWAY OUTPUT !!!

GATEWAY_MAC = b'\xcc\x50\xe3\x93\x0f\x58'  # Example: b'\x24\x6f...'

WIFI_CHANNEL = 3  # MUST match the Gateway's channel

# ----------------------------------------

# Setup WLAN

sta = network.WLAN(network.STA_IF)

sta.active(True)

sta.config(dhcp_hostname=DEVICE_NAME)

# Set the channel to match Gateway

sta.config(channel=WIFI_CHANNEL)

sta.disconnect() # Ensure we are not connected to any AP

print(f"Node started on Channel {WIFI_CHANNEL}")

# Setup ESP-NOW

e = espnow.ESPNow()

e.active(True)

try:

e.add_peer(GATEWAY_MAC)

except OSError:

print("Peer already exists or invalid MAC")

# Setup Sensor

adc = ADC(Pin(SENSOR_PIN))

adc.atten(ADC.ATTN_11DB)

adc.width(ADC.WIDTH_12BIT)

def read_humidity():

raw = adc.read()

# Map 0-4095 to 0-100%

return round((raw / 4095.0) * 100, 2)

def send_data(val):

payload = {

"nodeid": NODE_ID,

"value": val

}

msg = json.dumps(payload)

try:

e.send(GATEWAY_MAC, msg)

print(f"Sent: {msg}")

except OSError as err:

if err.args[0] == 116: # ESP_ERR_ESPNOW_NOT_FOUND

print("Error: Gateway not found (Check Channel/MAC)")

else:

print("Send Error:", err)

while True:

hum = read_humidity()

send_data(hum)

time.sleep_ms(READ_INTERVAL)

----------------------------------------------------------------------------------------------------------------